Electronics and Electrical Communications Engineering
Department

Faculty of Engineering

Cairo University

Submitted to: Dr. Omar Nasr

Adaptive Energy Compaction & Spatial

Entropy Mapping

Final Report submitted for course ELC448 “Digital Signal Processing“

4th Year

1st Semester - Academic Year 2025/2026

Prepared by:

| NAME | SECTION | ID |
| --- | --- | --- |
| Yousef Khaled Omar Mahmoud | 4 | 9220984 |

Submission Date: 13 February 2026

# Table of Contents

1.	Table of Contents	2

2.	Table of Figures	4

3.	List of Tables	4

5.	Executive Summary	6

6.	Name of IoT Vertical Application	6

7.	Functions Performed by the IOT System	6

7.1	 Real Time Parking Slot Occupancy Detection	7

7.2	 Multi Slot Data Collection and Local Processing	7

7.3	 Hierarchical Data Aggregation (Star Topology)	8

7.4	 Cloud Based Parking Availability Management	9

7.5	 Automated Gate Control Based on Availability	10

7.6	 Mobile Application User Guidance	11

7.7	 Alerting and Notification System	12

7.8	 System Scalability and Expansion	13

7.9	 Summary of System Functions	14

8.	TYPE OF DATA COLLECTED	14

8.1	Parking Slot Sensor Data	15

8.2	 Parking Slot Event Data	15

8.3	 Gateway Aggregated Data	16

8.4	Central Node to Cloud Data	16

8.4.1	 Example JSON Slot Snapshot	16

8.5	Transmission Protocol Summary	16

8.6	Daily Data Volume Estimation	17

8.7	Cloud Download Data	17

8.8	 Bandwidth & Storage Evaluation	18

9.	Information Presentation to End-User	19

9.1	 Central Node Touchscreen Dashboard (On-Site Interface)	19

9.2	Mobile Application Interface (Remote User Access)	20

9.3	AI Conversational Assistant	21

9.4	Alert Notification & Escalation System	22

10.	Detailed Schematic Diagram for the Overall System	23

10.1	Physical Deployment Layout	24

10.2	Zonal Partitioning	25

10.3	 Sensor Placement and Radio Coverage	26

10.3.1	Wireless Coverage Constraints	26

10.4	 Central Node & Backhaul Connectivity	27

10.5	System Architecture	27

10.6	Type and Location of Sensors	28

10.7	Power Budget and Energy Model	29

10.8	Type & Location of Actuators	30

10.9	Type & Location of Gateways — Network Layer Distribution	30

10.10	 Communication Technique — Sensor Nodes → Gateway	31

10.11	Backend Servers for Storage & Processing	31

11.	Hardware Components: Brand, Model, and Datasheets	31

11.1	Parking Slot Sensors	32

11.1.1	Infrared Occupancy Sensor	32

11.1.2	Ultrasonic Distance Sensor (Redundancy Check)	32

11.2	Sensor Node Controller	33

11.2.1	Microcontroller Unit (MCU)	33

11.3	Gateway Hardware	33

11.3.1	Parking Zone Gateway	33

11.4	Central Node (Edge Controller)	34

11.4.1	Central Processing Unit	34

11.5	Summary of Hardware Components	35

12.	Software Implementation of the Smart Parking IoT System	35

12.1	Software Architecture Overview	35

12.2	Sensor Node Software (Embedded Firmware)	35

12.2.1	Firmware Responsibilities	36

12.2.2	 Sensor Node Pseudocode	36

12.3	Gateway Software Implementation	37

12.3.1	Gateway Responsibilities	37

12.3.2	Gateway Pseudocode	38

12.4	Central Node Software Implementation	38

12.4.1	 Central Node Responsibilities	39

12.5	Cloud-Side Software Services	39

12.5.1	Cloud Functions	39

12.6	Security and Reliability Mechanisms	40

12.7	Summary	40

13.	Security Services for Safe Operation of the Smart Parking IoT System	40

13.1	Security Threat Model	40

13.2	 Device-Level Security (Sensor Nodes)	41

13.3	Gateway, Cloud and Application Security	41

13.4	Security Summary	43

14.	Prototype Implementation & Hardware Validation	43

14.1	Prototype Overview	43

14.2	Gate Monitoring Subsystem (Vehicle Counting)	44

14.3	Parking Slot Sensing Subsystem	45

14.4	 Gateway Aggregation Subsystem	46

14.5	Communication Validation	46

14.6	Prototype Limitations and Scalability	46

15.	References	47

16.	APPENDIX A — FULL SOFTWARE SOURCE CODE	48

16.1	Sensor Node Firmware (ESP32)	49

16.1.1	Functional Responsibilities	49

16.1.2	Firmware Source Code	49

16.2	Gateway Aggregation Software	50

16.2.1	Gateway Responsibilities	50

16.2.2	Gateway Source Code	50

16.3	Central Node Software	51

16.3.1	Central Node Responsibilities	51

16.3.2	Central Node Source Code	51

17.	Appendix B: Acronyms and Abbreviations	52

# Table of Figures

Figure 1 Parking Slot Occupancy Detection [1]	7

Figure 2 Sensor Nodes Aggregation per Parking Zone [1]	7

Figure 3 Star Topology Architecture [1]	8

Figure 4 Cloud Backend Processing and Storage [1]	9

Figure 5 Automatic Gate Control Workflow [1]	10

Figure 6 Mobile Application Interface Workflow [1]	11

Figure 7 Event and Alert Generation [1]	12

Figure 8 Scalable Smart Parking Deployment [1]	13

Figure 9 Functional Overview Diagram [1]	14

Figure 10 Parking Slot Sensor Data Frame Format [1]	15

Figure 11 Parking Slot Event Data Frame Formate [1]	15

Figure 12 Gateway Aggregated Data Frame Format [1]	16

Figure 13 Central Node to Cloud Frame Format [1]	16

Figure 14 System-Wide Data Flow & Storage Evaluation [1]	18

Figure 15 Central Node Touchscreen Dashboard  [1]	19

Figure 16 AI Assistant Interaction Flow [1]	21

Figure 17 Alert Escalation Logic [1]	23

Figure 18 Physical Deployment Layout [1]	24

Figure 19 Sensor Placement and Radio Coverage [1]	26

Figure 20  Central Node & Backhaul Connectivity [1]	27

Figure 21 Three-Tier System Architecture [1]	28

Figure 22 Communication Technique Across System Layers [1]	31

Figure 23 Firmware Execution Flow [1]	37

Figure 24 Gateway Execution Flow [1]	38

Figure 25 Central Node Execution Flow [1]	39

Figure 26 Device & Communication Security [1]	41

Figure 27 End-to-End Security Architecture [1]	42

Figure 28 Gate Monitoring Subsystem [1]	44

Figure 29 Parking Slot Sensing Subsystem [1]	45

Figure 30 Gateway Aggregation Subsystem [1]	46

# List of Tables

Table 1 Parking Slot Sensor Data Specifications [1]	15

Table 2 Parking Slot Event Data [1]	15

Table 3 Gateway Aggregated Data [1]	16

Table 4 Central Node to Cloud Data Transmission [1]	16

Table 5 MQTT Advantages [2]	17

Table 6 Daily Data Volume [3]	17

Table 7 Cloud Download [4]	17

Table 8 System-Wide Data Flow & Storage [5]	18

Table 9 Hardware Configuration of On-Site HMI [DataSheet] [6]	20

Table 10 Software Architecture [7]	20

Table 11 Alert Notification Channels [1]	22

Table 12 Automated Intervention Examples [1]	22

Table 13 Zone Description [1]	25

Table 14  Sensor Location [8]	28

Table 15 Baseline Power Characteristics [8] [9]	29

Table 16 Actuator Location [1]	30

Table 17 Gateway Function [1]	30

Table 18 Backend Services [3] [4]	31

Table 19 IR Parking Occupancy Sensor [10]	32

Table 20 Ultrasonic Distance Sensor [11]	32

Table 21 Sensor Node Controller [12]	33

Table 22 Gateway Node Hardware [13]	33

Table 23 Central Node Hardware [14]	34

Table 24 Hardware Summary [1]	35

Table 25 Platform and Development Environment [1]	35

Table 26 Platform and Environment [1]	37

Table 27 Platform and Stack [1]	38

Table 28 Cloud Platform [1]	39

Table 29 Acronyms and Abbreviations[1]	52

# Executive Summary

Traffic congestion inside urban areas is significantly aggravated by drivers roaming parking facilities in search of available parking spaces. This behavior increases fuel consumption, travel time, and local emissions, while also creating unnecessary congestion near commercial centers, universities, and residential complexes.

This project presents the design of an IoT-based Smart Parking System that enables real-time monitoring of parking slot occupancy, remote user access through a mobile application, and automated gate control based on parking availability. The system aims to minimize search time for parking spaces by providing drivers with accurate, up-to-date information about free and occupied slots before and during arrival.

The proposed system deploys infrared (IR) proximity sensors at each parking slot to detect vehicle presence. These sensors are connected to local IoT nodes, which transmit occupancy data to a gateway controller. The gateway aggregates slot data and forwards it to a cloud backend using standard IoT communication protocols. A mobile application interface allows users to visualize parking availability in real time and guides them directly to free spaces. In addition, an electromechanical gate actuator is integrated at the parking entrance to automatically permit or deny access depending on slot availability.

The system is designed to be scalable, cost-effective, and easy to deploy, making it suitable for parking facilities of varying sizes. Emphasis is placed on clear physical deployment, well-defined communication paths, and standardized software protocols to ensure reliability and maintainability.

This report focuses on the system architecture, physical layout, sensor and actuator deployment, communication techniques, backend infrastructure, and man-machine interface, supported primarily through schematic figures and diagrams to clearly demonstrate system operation and engineering decisions.

# Preprocessing Results

Table 1 Image dimensions after zero-padding and cropping for different block sizes

| Block Size (n) | Image | Original Size (H × W) | Zero-Padded Size (H × W) | Cropped Size (H × W) |
| --- | --- | --- | --- | --- |
| 8 | 1.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 2.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 3.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 4.png | 512 × 768 | 512 × 768 | 512 × 768 |
| 16 | 1.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 2.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 3.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 4.png | 512 × 768 | 512 × 768 | 512 × 768 |
| 32 | 1.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 2.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 3.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 4.png | 512 × 768 | 512 × 768 | 512 × 768 |
| 64 | 1.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 2.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 3.png | 512 × 768 | 512 × 768 | 512 × 768 |
|  | 4.png | 512 × 768 | 512 × 768 | 512 × 768 |

As seen in Table 1, All images have dimensions that are already divisible by the tested block sizes (8, 16, 32, and 64). Therefore, both zero-padding and cropping operations result in identical image dimensions, ensuring that no artificial padding or spatial information loss is introduced during preprocessing. This allows subsequent DCT-based analysis to focus solely on transform and truncation effects.

# Functions Performed by the IoT System

The Smart Parking System performs multiple integrated functions that enable real-time parking monitoring, user guidance, and automated access control. The core system functions are illustrated primarily through schematic figures, with concise explanations provided below.

Figure 1 Parking Slot Occupancy Detection [1]

Real Time Parking Slot Occupancy Detection

As shown in Figure 1, each parking slot is equipped with an infrared (IR) proximity sensor mounted at ground or curb level. The sensor continuously detects the presence or absence of a vehicle in the slot.

This function eliminates manual inspection and enables continuous, automated monitoring of the entire parking area.

# Appendix B: Acronyms and Abbreviations

Table 29 summarizes all acronyms used throughout this report for quick reference.

Table 2 Acronyms and Abbreviations[1]

| Acronym | Full Meaning / Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| AES | Advanced Encryption Standard |
| AI | Artificial Intelligence |
| API | Application Programming Interface |
| BLE | Bluetooth Low Energy |
| CAT6 | Category 6 Ethernet Cable |
| CPU | Central Processing Unit |
| CRC | Cyclic Redundancy Check |
| DB | Database |
| DoS | Denial of Service |
| ESP | Espressif Microcontroller Platform |
| ESP32 | Wi-Fi & Bluetooth SoC by Espressif Systems |
| FCS | Frame Check Sequence |
| GPIO | General Purpose Input/Output |
| HMI | Human–Machine Interface |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | Hypertext Transfer Protocol Secure |
| IoT | Internet of Things |
| IP | Internet Protocol |
| IP65 | Ingress Protection Rating (Dust-tight & Water-resistant) |
| IR | Infrared |
| ISM | Industrial, Scientific and Medical (radio band) |
| JSON | JavaScript Object Notation |
| JWT | JSON Web Token |
| LAN | Local Area Network |
| LoRa | Long Range Radio Communication |
| LTE | Long Term Evolution (4G Cellular) |
| MAC | Medium Access Control |
| MCU | Microcontroller Unit |
| MIC | Message Integrity Code |
| MQTT | Message Queuing Telemetry Transport |
| NFC | Near Field Communication |
| OS | Operating System |
| PCB | Printed Circuit Board |
| QoS | Quality of Service |
| RAM | Random Access Memory |
| RBAC | Role-Based Access Control |
| REST | Representational State Transfer |
| RF | Radio Frequency |
| RTOS | Real-Time Operating System |
| SBC | Single Board Computer |
| SN | Sensor Node |
| SoC | System on Chip |
| SQL | Structured Query Language |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| UART | Universal Asynchronous Receiver–Transmitter |
| UI | User Interface |
| USB | Universal Serial Bus |
| Wi-Fi | Wireless Fidelity |
| WS | WebSocket |
| X.509 | Public Key Infrastructure Certificate Standard |